MQTT Technical Documentation

Technical reference covering Mosquitto administration, OpenSSL TLS/mTLS setup, Paho Python client architecture, and error handling.

1. Core Concepts & Architecture

MQTT: A lightweight, low-bandwidth publish/subscribe messaging protocol designed for constrained devices and high-latency networks.

Publisher: A client device or application that sends messages to a specific topic.

Subscriber: A client device or application that receives messages by subscribing to topics.

Broker: A central server that routes messages between publishers and subscribers; clients never communicate directly with each other.

Topic: A UTF-8 routing string/channel that publishers and subscribers use to address messages.

2. Mosquitto Broker Management (Windows)

Service Commands

net start mosquitto: Starts the Mosquitto broker as a Windows background service.

mosquitto -c mosquitto.conf: Runs the broker in the foreground using a specified configuration file (useful for debugging).

net stop mosquitto: Stops the running Mosquitto Windows service.

net stop mosquitto && net start mosquitto: Restarts the service to apply changes made to mosquitto.conf.

Capacity & Connection Controls (mosquitto.conf)

max_connections: Limits the total simultaneous connections allowed for a specific listener/port.

global_max_connections: Limits the total connections permitted across all listeners combined.

global_max_clients: Restricts the total number of client IDs the broker tracks in memory.

3. Security: TLS & Mutual TLS (mTLS)

TLS encrypts traffic over the wire. Mutual TLS (mTLS) requires both the broker and the client to present valid X.509 certificates to authenticate each other's identity.

OpenSSL Certificate Chain Generation

# 1. Generate Encrypted Root CA Key & Certificate
openssl genrsa -des3 -out ca.key 2048
openssl req -new -x509 -out ca.crt -key ca.key -days 365

# 2. Generate Server Key & CSR (with non-interactive -subj)
openssl genrsa -out server.key 2048
openssl req -new -out server.csr -key server.key -subj "/CN=domain"

# 3. Sign Server Certificate (Includes SAN extension from san.cnf)
openssl x509 -req -out server.crt -in server.csr -CA ca.crt -CAkey ca.key -CAcreateserial -days 365 -extfile san.cnf -extensions v3_req

# 4. Generate Client Key, CSR, & Sign Certificate
openssl genrsa -out client.key 2048
openssl req -new -out client.csr -key client.key -subj "/CN=client1"
openssl x509 -req -out client.crt -in client.csr -CA ca.crt -CAkey ca.key -CAcreateserial -days 360

Subject Alternative Name (SAN) Configuration (san.cnf)

[v3_req]
subjectAltName = @alt_names

[alt_names]
IP.1 = 10.58.166.132
IP.2 = 127.0.0.1
DNS.1 = localhost

4. Mosquitto TLS Listener Configuration

listener 8883
cafile C:/Program Files/Mosquitto/ca.crt
certfile C:/Program Files/Mosquitto/server.crt
keyfile C:/Program Files/Mosquitto/server.key
require_certificate true
use_identity_as_username true
allow_anonymous false

listener 8883: Standard network port for MQTT over TLS.

require_certificate true: Enforces mTLS authentication by requiring client certificate presentation.

use_identity_as_username true: Maps the Common Name (CN) embedded in the client certificate directly as the MQTT username (e.g., client1).

5. Terminal CLI Testing Commands

# Subscriber Test with mTLS Credentials
mosquitto_sub --cafile ca.crt --cert client.crt --key client.key -h 10.58.166.132 -p 8883 -t test

# Publisher Test with mTLS Credentials
mosquitto_pub --cafile ca.crt --cert client.crt --key client.key -h 10.58.166.132 -p 8883 -t "test" -m "hello there"

6. Python Paho MQTT Client Implementation

Event Callbacks Architecture

| Callback Name | Signature | Purpose / Flow Description |
| --- | --- | --- |
| on_connect | (client, userdata, flags, reason_code, properties) | Triggered on TCP connection establishment. Evaluates reason_code.is_failure before executing subscribe(). |
| on_subscribe | (client, userdata, mid, reason_codes, properties) | Broker acknowledgment for subscription. Triggers subsequent logic like publish(). |
| on_publish | (client, userdata, mid, reason_code, properties) | Acknowledges message delivery confirmation from the broker. |
| on_message | (client, userdata, message) | Executes when a message arrives. Decodes message.payload.decode('utf-8') with a fallback to repr(). |
| on_disconnect | (client, userdata, disconnect_flags, reason_code, properties) | Executes upon connection termination or network drop. |

Network Loop Execution Models

loop_forever(): Blocking call on the main thread. Manages network traffic, callback execution, and handles automatic reconnection.

loop_start(): Runs the network loop asynchronously on a background thread. Ideal for applications multi-tasking or managing multiple client instances.

7. Return Codes & Diagnostics

reason_code 0: Connection Success.

reason_code 1: Unacceptable protocol version.

reason_code 2: Client identifier rejected by broker.

reason_code 3: Broker/Server unavailable.

reason_code 4: Bad username or password.

reason_code 5: Unauthorized connection.

ssl.SSLCertVerificationError: Certificate/Key mismatch, expiration, or host mismatch during handshake.

ConnectionRefusedError: Mosquitto service is stopped or port 8883 is blocked/wrong.

8. Learning Backlog & Advanced Features

Quality of Service (QoS) Levels: Practical implementation of QoS 0 (at most once), QoS 1 (at least once), and QoS 2 (exactly once).

Retained Messages: Managing topic flags so the broker stores the last valid message for new subscribers.

Last Will and Testament (LWT): Configuring client drop notices sent automatically by the broker upon ungraceful client disconnection.

Persistent Sessions: Setting clean session flags to preserve message queues for offline subscribers.

MQTT 5.0 User Properties: Utilizing custom user properties in publish packets and callbacks.